DIMENSIONING PRINCIPLES

The overriding principle of dimensioning is CLARITY.

- Each feature of an object is dimensioned once, and only once.

- Dimensions should be selected to suit the function of the object

- Dimensions should be placed in the most descriptive view of the feature being dimensioned.

- Dimensions should specify only the size of the feature.  The manufacturing method should be specified only if it is a mandatory design requirement.

- Angles shown as right angles on drawings are assumed to be 90 degrees unless otherwise specified, and need not be dimensioned.

- Dimensions should be moved outside the boundaries of the object whenever possible.

- Dimension lines should be aligned and grouped where possible to promote clarity and uniform appearance.

- Crossing extension and dimension lines should be avoided whenever possible.

- The space between the first dimension line and the object should be at least ½” (inch).

- The space between the dimension lines should be at least 3/8” (inch).

- There should be a visible gap between the object and the origin of an extension line.

- Extension lines should extend 1/8” (inch), beyond the last dimension line.

- Extension lines should be broken if they cross, or are close to, arrowheads.

- Leader lines used to dimension circles or arcs should be radial with a perfect angle of 60 degrees.

- Dimensions should be oriented to read from the bottom of the drawing.

- Diameters are dimensioned with a numerical value and the diameter symbol (zero with a slash through it, or D for diameter).

- Concentric circles (cylinders), should be dimensioned in a longitudinal (rectangular), view where possible.

- Radii are dimensioned with a numerical value preceded by a radius symbol ( R ).

- When a dimension is given to, or through, the center of an arc, a small cross is shown at the center (which is the result of the mid point of two center-lines crossing).

- The diameter and depth of holes that are counter-bored, spot-faced, or counter-sunk should be specified in a note.

- The depth of a blind hole may be specified in a note, and is the depth of the full diameter from the surface of the object.